| Student Name | : | Tan Yi Rong |
| --- | --- | --- |
| Student ID Number | : | U2320967K |
| Company Name | : | GlobalFoundries APAC |
| Company Supervisor | : | Ng, Christopher |
| Faculty Supervisor | : | Upadrasta Ramamurty |
| Date of Submission | : | Click or tap to enter a date. |

Student Journal #1 Assessment Submission

Please note that the prompts provided in this document are meant as guides to help you start the reflection process. They are not purposed to be exhaustive list of items to consider.

- Discuss - your internship organisation’s nature of business, and its organisational culture. GlobalFoundries (GF) is a leading global semiconductor foundry that provides advanced manufacturing services for integrated circuits. Operating on a pure-play foundry model, GF manufactures chips designed by other companies. The company fosters a culture of innovation, collaboration and continuous improvement, with a strong emphasis on safety, quality and operational excellence. GF offers a wide range of semiconductor manufacturing technologies, including radio frequency (RF), analog and embedded memory solutions. Its services span from design enablement to wafer fabrication and testing. GF serves global markets, with key customers in automotive, mobile, IoT and data center sectors. Suppliers include equipment manufacturers and raw materials providers while competitors include TSMC, Samsung Foundry, UMC and Intel. GF’s competitive edge by focusing on specialised process technologies, advanced packaging and a strong emphasis on collaboration and innovation. GF APAC also fosters a supportive and inclusive work environment that emphasises both professional development and employee well-being. It has weekly sports activities, events that celebrate different cultures and company team bonding cohesions. As an intern in the Fab7 Automated Material Handling System (AMHS) department, I’ve experienced a welcoming and team-oriented atmosphere. Senior engineers and mentors are approachable and always willing to share their knowledge. Even when they are busy, they make the effort to educate me on areas where I’m lacking in this industry, which has greatly enhanced my learning journey. I also appreciate the inclusive and friendly environment. I’m regularly invited to join my colleagues for breakfast breaks and lunch, and I never feel left out. Instead, I feel genuinely welcomed and part of the team.

- Describe the problem(s) that the internship organisation wants you to address and your analysis towards addressing the problem(s). Briefly describe the problem(s) provided by the internship organisation; and the expected scope of work and deliverables expected from you to address the issues. Provide your assumptions and analysis to derive the problem root cause; and your suggested approach to address the issue. Share about possible enablers and obstacles towards solving the problem(s). The main problem that my internship company has tasked me to address is the automation of the manual measurement process used to assess the wear and tear of the Overhead Hoist Transport (OHT) travel wheels. Currently, line support engineers manually measure the diameter of the OHT wheels using vernier caliper to determine whether the rubber tire needs replacement or if a shim adjustment is necessary. However, this process is prone to human error. Variations in pressure or measurement angle can lead to inconsistent and inaccurate readings. This is evident in the collected data, where some measurements appear to increase over time, contradicting the expected wear due to friction. These inconsistencies highlight the need for a more reliable and consistent method. To address this, the AMHS department aims to implement an automated system using sensors to obtain effectively obtain accurate measurements. Additionally, the goal is to integrate machine learning techniques to predict remaining lifespan of the tires and flag OHTs that require maintenance before major issues arise. This predictive maintenance approach would improve accuracy and reduce unplanned downtimes. My role involves analysing the current process, identifying suitable sensor technologies, developing a prototype system and exploring predictive analytic models.

- Derive the solution to solve the assigned problem(s). Explain your solution and provide an illustrated plan and detail the steps that you are taking to address the problems(s). Describe your analysis towards addressing the problem statement, your assumptions and solution to the problem. Explain the steps to provide the solution and the types of resources that you might need to accomplish your goals. To address the issue of inconsistent manual measurements of OHT travel wheels, our project team, led by two senior engineers and supported by myself, is developing an automated measurement system using sensors. The objective is to replace the current manual process with a more accurate and consistent method. In the long run, we aim to integrate machine learning techniques to predict tire wear and enable predictive maintenance. This solution is intended to reduce human error, improve data reliability and minimize unplanned downtimes. The project is currently in its first phase, which involves ideation, sensor procurement and prototype design. As a team, we began by conducting on-site observations of the OHT system and maintenance workflow to better understand the operational and environmental constraints. These observations helped us define the technical requirements for the sensor system such as measurement precision, sensor placement and future implementation. We then engaged with multiple vendors to explore different sensor technologies including laser and vision-based sensors. The vendors have been highly responsive, providing personal recommendations and product comparisons which greatly helped us with the evaluation of the sensors. Once a suitable sensor is selected and tested, the next step will be to collect real-time measurement data and compare it with historical records to validate accuracy. This data will also serve as the foundation for training a machine learning model that can predict the remaining lifespan of the OHT tires. The final solution will include a dashboard or alert system to notify engineers when an OHT is approaching its maintenance threshold, allowing for timely interventions. This project requires a range of resources, including access to the OHT for testing, collaboration with vendors and internal engineering teams, computing infrastructure for data processing and model training and sufficient time to meet deadlines. While vendor support has been a key enabler, scheduling meetings has occasionally been challenging due to the project leads’ responsibility on the support line. Time is also a constraint, as the project is expected to continue into my Professional Internship (PI) and is projected to be completed by December. Another challenge is evaluating the trade-offs between different sensor technologies to ensure the chosen solution is both technically feasible and realistic within the project timeline.

- Report your current state of progress - how much progress have you made, what is helping you to advance and/or impeding your advances. What kind of mitigation actions will you need to help improve your situation and/or your next step? Describe how you have been advancing with your plan and explain the help or impediment encountered as you are advancing in your plan. If you need an alternative plan or resources, explain what you will need and how would you go about obtaining those resources. At this stage of the project, we are in the early part of the implementation phase. Our team has completed the ideation and planning stages and is now focused on sensor procurement and prototype development. We are currently evaluating two vendors, Keyence and Omron, both of whom have been highly responsive and supportive. The decision between the two depend on several factors, including pricing, sensor functionality and compatibility with our data collection system. To support this evaluation, we are planning to request a T-loan from both companies which will allow us to conduct trial prototyping tests using their sensors in our environment. As part of this phase, we are also preparing to take precise measurements of the installation area where the sensor will be mounted. This is a critical step to ensure that the design fits seamlessly into the existing OHT infrastructure. I will be responsible for designing the mounting and housing components using CAD software with the goal of creating a manufacturable model that meets all mechanical and spatial requirements. To ensure the design is structurally sound and does not require rework after fabrication, I will also be performing Free Body Diagram (FBD) calculations. These calculations will help verify that the net force acting on the mounting frame is zero, ensuring stability and preventing the need for additional modifications or remanufacturing later on. We are considering parallel testing of both sensor options to accelerate the prototyping process. If needed, we may seek additional support from other senior engineer or line support engineer to help assist with testing, data logging or design validation. The next immediate steps include finalising the sensor selection, completing the design for the sensor mount, and beginning prototyping and testing phase.

- What have you learned from the experience and what are the shortcomings that could be better managed? Explain the progression of your project and whether you are meeting the outcome expected by the internship organization. What are the key learning points and areas that you could have perform in a better way? What is impeding your performance and how you may address these impediment(s). As this is my first internship, the experience has been truly eye opening. It has given me a much deeper appreciation of how industry projects operate in real-world setting. I have come to understand that even seemingly small details such as sensor placement, measurement accuracy or communication with vendors, can have a significant impact on the success of a project. This experience has taught me that in engineering, we cannot afford to overlook or slack on any factor, as each decision contributes to the overall reliability and functionality of the system. While we are still in the early implementation phase, the project has progressed steadily and is aligned with expectations set by the internship organization. Our team has completed the planning and vendor evaluation stages and is now preparing for prototyping and testing. Working alongside my two senior engineers has shown me the importance of collaboration as we often bounced ideas off one another to explore different perspectives and refine our approach. This open exchange of ideas allows us to evaluate the strengths and weaknesses of each proposal and come to an aligned conclusion on why a particular method or solutions is most suitable for the project. A key part of our workflow involves presenting our discussions and decisions to our supervisor, who provides valuable feedback and suggestions. His insights often help us refine our ideas and improve the direction of the project. This feedback loop has been instrumental in helping us stay aligned with the goal and continuously improve our approach. Technically, I have gained hands-on experience with sensor evaluation, mechanical design using Sketchup and SolidWorks for visualisation and FBD calculations to ensure structure stability. These are skills that I’ve learnt in NTU course such as MA2005 Engineering Graphics and MA2001 Mechanics of Materials. But designing it from scratch and applying the knowledge in a real-world context has been challenging as I must ensure that measurements, weight and external factors are all properly accounted for. That said, I recognise areas where I could have performed better. One of the key challenges has been time management, which has been affected not only by project-related tasks but also by additional responsibilities assigned by other senior engineers and the HR team. As an intern, I have been required to attend orientation sessions and training modules to gain foundational understanding of the semiconductor industry. Coming from a mechanical engineering background, the semiconductor field is entirely new to me. I have had to pick up relevant knowledge along the way through e-learning modules and internal lessons, which has added to my workload and will need to balance the priorities. Overall, this internship has been a valuable learning journey. It has not only strengthened my technical foundation but also helped me develop a more professional mindset when it comes to teamwork, accountability and attention to detail.

- Additional Journal Questions (Non-Gradable)

| No |  | Yes | No |
| --- | --- | --- | --- |
| 6.1 | I am adequately supported in my internship workplace. | □ | □ |
| 6.2 | I am adapting well in my internship workplace. | □ | □ |
| 6.3 | I am applying my learning into this internship. | □ | □ |
| 6.4 | This internship reinforces what I want as a career when I graduate from NTU. | □ | □ |
| 6.5 | The employer has expressed an interest to offer me a job after my graduation. | □ | □ |
| 6.6 | I am interested to work in this organisation after my graduation. | □ | □ |