# Bill of Materials (BOM) — OHT PM Automation Project

This document is formatted for procurement and finance, broken down into two distinct purchasing phases to minimize financial risk.

## 1. Prototype Phase (De-Risking & Proof of Concept)

Objective: To validate the electronic signal integrity of the magnetic connectors and custom relay switching board before purchasing high-value robotics.

| Company / Vendor | Item Name | Qty | Unit Price (Est. SGD) | Total Price (Est. SGD) | Remarks |
| --- | --- | --- | --- | --- | --- |
| AliExpress / Taobao | Generic 15-Pin Magnetic Pogo Connectors | 5 Pairs | $15.00 | $75.00 | Cheap alternative for bench testing. Extra spares included. |
| Rosenberger | Premium 15-Pin Magnetic Connector | 2 Pairs | $70.00 | $140.00 | Premium benchmark for signal integrity testing vs the generic plugs. |
| Sim Lim Sq / Shopee | 4-Channel 5V Relay Module (Optoisolated) | 4 | $8.00 | $32.00 | To switch between the 4 sensor channels. Spares included. |
| Sim Lim Sq / Shopee | 5mm LED Assortment & Resistors | 1 Pack | $5.00 | $5.00 | Crucial for visually proving to stakeholders which sensor channel is active during the test. |
| Sim Lim Sq / Shopee | Raw Perfboards & Breadboards | 1 Set | $5.00 | $5.00 | For quick-and-dirty, instant hand-wired testing of the magnetic connection. |
| Sim Lim Sq / Shopee | ESP32 Microcontroller Board | 5 | $12.00 | $60.00 | Logic controller. Extras included as these are prone to shorting out during prototyping. |
| JLCPCB | Custom PCB Fabrication (Breakout & Relay Boards) | 1 Batch | $20.00 | $20.00 | Ordered only after the raw perfboard tests prove successful. |
| Misumi / RS Components | 2020 Aluminum Extrusion, Brackets & M4 T-Nuts | 1 Set | $50.00 | $50.00 | To build a physical tabletop mockup of the OHT port interface. |
| SUBTOTAL |  |  |  | ~$387.00 | Low-risk validation budget. |

## 2. Full-Scale Implementation Phase (Production Station)

Objective: Final hardware build for the automated workstation, assuming Phase 1 prototype successfully validates the connection logic.

| Company / Vendor | Item Name | Qty | Unit Price (Est. SGD) | Total Price (Est. SGD) | Remarks |
| --- | --- | --- | --- | --- | --- |
| ABB | ABB PoWa Collaborative Robot | 1 | $20,000.00 | $20,000.00 | Primary robotic arm. Awaiting final vendor quotation. |
| SICK | PLOC2D-611-6RB Robot Guidance System (PN: 1136307) | 1 | $3,900.00 | $3,900.00 | 1.2MP, 0.23mm accuracy. Verify integration compatibility with ABB PoWa. |
| Rosenberger | Premium 15-Pin Magnetic Connector (OHT Retrofit) | 305 | $70.00 | $21,350.00 | 304 for fleet retrofit, 1 for the robot end-effector. |
| JLCPCB / PCBWay | Custom Breakout PCBs (OHT Retrofit) | 305 | $2.00 | $610.00 | Final professional PCBs to mount the magnetic plug to the 304 OHTs. |
| Rexroth / Local Fab | Heavy-Duty 8080 Aluminum Enclosure / Cage | 1 | $800.00 | $800.00 | To securely house the cobot and protect it from accidental bumps by line support. |
| Generic Automation | Linear Actuator & Controller (For Panel Shifting) | 1 | $250.00 | $250.00 | For Phase 2 automation of the obstacle simulation panel. |
| SUBTOTAL |  |  |  | ~$46,910.00 |  |

### Procurement Notes for Finance:

- Vision System Selection: Two SICK options were provided. We have selected the $3,900 PLOC2D-611-6RB model. Because we are using self-aligning magnetic plugs, the 0.23mm accuracy of this 1.2MP camera is more than sufficient for coarse positioning. The $7,000 (5.1MP) model is unnecessary overkill for this specific mechanical design. Note: The quotation references the ABB GoFa, so compatibility with the ABB PoWa must be verified with the vendor.

- Staged Purchasing: The $155 Prototype Phase must be purchased and assembled first. The $46,000 Full-Scale Phase should remain locked until the engineering team signs off on the signal integrity of the prototype board.

## 3. Hardware Evaluation & Specifications

To justify the chosen components, the engineering alternatives and their respective specifications are documented below.

### A. SICK Smart Vision Camera Comparison

The camera is required for coarse 2D robotic guidance. Because the chosen magnetic plug is self-aligning upon contact, ultra-high-resolution vision is not required.

| Specification | Option 1: PLOC2D-611-6RB (Selected) | Option 2: PLOC2D-8305-12 (Rejected) |
| --- | --- | --- |
| Sensor Resolution | 1.2 MP (1,280 x 960 px) | 5.1 MP (2,464 x 2,048 px) |
| Working Distance | 150 mm | 100 mm |
| Field of View (X, Y) | 60 mm x 44 mm | 56 mm x 46 mm |
| Guidance Accuracy | 0.2344 mm | 0.11365 mm |
| Quoted Price | SGD 3,900 | SGD 7,000 |
| Status / Rationale | APPROVED - Accuracy of 0.23mm is well within the 3-5mm capture zone of a standard magnetic plug. Most cost-effective. | REJECTED - Double the price for extreme precision (0.11mm) that is mechanically wasted when using a self-aligning magnet. |

### B. ABB Collaborative Robot (Cobot) Comparison

The robotic arm acts as the primary motion mechanism to move the sensor plug to the OHT.

| Specification | ABB PoWa (Selected) | ABB GoFa (Alternative) |
| --- | --- | --- |
| Estimated Price | ~SGD 15,000 – 20,000 | ~SGD 20,000 – 30,000 |
| Reach | Up to 1300mm | Up to 1300mm |
| Target Application | General automation and handling | High-speed, high-precision collaboration |
| Status / Rationale | APPROVED - Lower price tier. Perfect for “blind” or coarse-guided movements where mechanical fixtures (like magnetic plugs) handle the final alignment. | REJECTED - Higher cost tier. Unnecessary for a task that has been mechanically de-risked by replacing standard pins with magnets. |