# Technical Requirements & RFQ: Sensor Switching Prototype Board

To: Kuriosity (Sim Lim Tower) From: Tan Yi Rong (GlobalFoundries Singapore, F7 AMHS) Project: OHT Sensor PM Automation (Prototype Phase)

## 1. System Architecture Overview

We are building a robotic automation test station for our Overhead Hoist Transport (OHT) fleet. The system architecture is split into two halves:

- The OHT Side (Passive): The 4 existing sensor ports (VHL, OBS Left, OBS Right, OBS Center) on the vehicle are wired into a simple passive PCB that merges them into 1 magnetic socket.

- The Workstation Side (Active): A collaborative robot holding a magnetic plug will snap into the OHT. A cable will run from this plug down to an Active Switching Board, which then connects to a laptop.

Scope of Work: We require you to build the prototype for the Active Switching Board (on perfboard/breadboard).

## 2. Key Technical Requirements

This board must take the single data line coming from the magnetic plug and route it sequentially to one of four output channels (which go to the laptop).

CRITICAL REQUIREMENT: NO Mechanical Relays Because we are routing sensitive waveform data from Hokuyo obstacle sensors, mechanical relay contact “bounce” and physical wear are unacceptable. We require this switching to be handled by an IC Switch / Analog Multiplexer (e.g., CD4051/CD4052 or 4x individual solid-state IC switches). The data must switch cleanly without signal shredding.

### Shielding & EMI Mitigation:

Due to the noisy electromagnetic environment of a semiconductor fab, please use shielded wiring/jumpers for the data lines on this prototype where feasible to prevent EMI noise.

### Logic Control:

- The IC switching must be controlled by an ESP32 Microcontroller.

- The laptop will command the ESP32 (via USB/serial) to cycle the active channel. The physical ESP32 needs to be mounted and wired to the IC switch logic pins.

### Visual Diagnostics:

- Please wire four 5mm LEDs (with resistors) to the circuit.

- Each LED should correspond to one of the 4 sensor channels. When the ESP32 activates Channel 2, LED 2 should light up. This is crucial for our visual proof-of-concept during stakeholder presentations.

## 3. Required Deliverables / Bill of Materials (BOM) to Quote

Please provide a quotation to supply the parts and perform the basic soldering/wiring assembly for the following on a prototype perfboard:

| Item Description | Qty Required | Remarks |
| --- | --- | --- |
| ESP32 Microcontroller Board | 1 | Logic controller to drive the IC switch. |
| IC Switch / Multiplexer Chip | 1 (or 4x discrete) | Must cleanly route sensor data without mechanical bounce. |
| 5mm LEDs + Resistors | 4 | Visual indicators for the 4 active channels. |
| Shielded Wiring & Perfboard | 1 Set | To securely mount and wire components while mitigating noise. |
| Header Pins / Terminals | 1 Set | 1x main input cluster and 4x output clusters. |
| Assembly / Wiring Labor | 1 | Cost to solder and wire the prototype into a working test unit. |

## 4. Next Steps

Please review these requirements and let me know: 1. Which specific IC switch you recommend for handling this serial sensor data. 2. The estimated cost for the parts and the assembly labor. 3. The estimated lead time to build this prototype.

Thank you.