# Presentation Report — Full Slide Contents

---

## Slide 1

Professional Attachment '26
Tan Yi Rong | Equipment Engineer Intern | F7 AMHS
Date: May 25, 2026 – Aug 7, 2026

GlobalFoundries Confidential
Any hard and soft copy out of the Document Management System deemed as uncontrolled

---

## Slide 2

Project ODIN
Calibrating the sensor
Creating a software app for data
01
FYP Research & Findings
Exploring project scope
Cost Findings
02

---

## Slide 3

# Project ODIN

PRIMARY GOAL
Develop an automated sensor-based monitoring system that measures OHT wheel wear condition in real-time to enable predictive maintenance and reduce unplanned downtimes.
Continuous Monitoring
The sensor captures live profile data of each wheel.
Any wear and tear over time will increase the height difference.
Data is logged for trend analysis for a predictive maintenance.
1st Detection
LiDAR Sensor
Direction of OHT
Laser
Continuous Scanning
Highest Point
1st Wheel
2nd Wheel
Track

---

## Slide 4

# Project ODIN CAD design
Overhead Hoist Transport (OHT)

Exact placement and mounting layout on OHT track

---

## Slide 5

# Project ODIN Layout
Power Supply
Ethernet Connectivity
Slave Amplifier Controller (Left)
Master Amplifier Controller (Right)
Circuit Breaker

---

## Slide 6

# Testing Phase
LiDAR Sensor

Measured the difference from the Original Wheel size of 125mm
Real Time Testing to simulate OHT running pass sensor
Detecting Highest point of the wheel

---

## Slide 7

# Part 1: Project ODIN

OBJECTIVE
Support Predictive Maintenance (PdM) through wheel condition monitoring.
01 Identify — Focused on identifying parameters for accurate PdM data collection
02 Execute — Executed continuous sensor testing to collect wheel measurement data
03 Validate — Validated sensor outputs to ensure they accurately reflected wheel degradation over time

---

## Slide 8

# Live Monitoring UI

Real-Time Data: Dashboard displaying live sensor reading with latest OHT No.

---

## Slide 9

# Fleet Overview UI

Overview Tracking: Displays the current status of the entire OHT fleet
Warning Flag: Highlights which vehicles are due for preventive maintenance

---

## Slide 10

# OHT Analysis UI

Trend Analysis: Graphs how wheel degradation progresses over weeks and months
Predictive Insights: Forecasts when specific wheels will fail, enabling pre-emptive parts replacement rather than reactive maintenance

---

## Slide 11

# Side-By-Side Wheel Comparison

Wheel Comparison: Enables detailed analysis of each wheel's degradation state for further investigation

---

## Slide 12

# Part 2: FYP Research & Feasibility

OBJECTIVE
Automate the repetitive and manual OHT Mark 2 Sensor PM Process
The current preventive maintenance (PM) process validates that the OHT sensors are functioning correctly and are within acceptable operating specifications before the unit is released for operation.
PURPOSE OF TESTING — What the PM process needs to confirm before an OHT returns for operation
01 Verify waveform and detection performance
02 Confirm readings are within required specification
03 Ensure safe braking / obstacle detection performance
04 Record proof of passing result before OHT is returned for operation

---

## Slide 13

VHL using Reflective Plate
OBS Right — Removable Reflective Plate
OBS Left
RN = VHL | JNR = OBS Right | JNL = OBS Left | JNC = OBS Center
10-pin male cable
Panel — OBS Center
Manual plugging and shifting of data port between sensors
Manual shifting of Panel
VHL – VeHicLe Sensor
OBS – OBStacle Sensor

---

## Slide 14

# Current PM Workflow
1 — Once finish VHL, remove reflective plate
2 — Shift panel to Left Side for No detect, then to right for Detect
For VHL — Adjust shifting of Panel
3 — Shift panel to Right Side for No detect, then to Left for Detect
4 — Shift panel down for No Detect, then raise it up for Detect
For OBS Left / For OBS Right / For OBS Center
Plug port in from left to right sequence
Ensure sensors are in correct specification, else manually adjust sensor position.
Once good, screenshot and upload readings
Change port when individual sensor readings are captured
Repeat Sequence from 1 to 4

---

## Slide 15

# Current Manual Process Challenges
PAIN POINTS IN THE EXISTING METHOD
The current workflow is effective, but it is highly dependent on manual execution.
Key challenges
- Repeated plugging and unplugging between ports
- Manual panel / reflective target shifting
- Repeated checking of G / NG result
- Manual sensor adjustment if out of specification
- Manual screenshot capture and upload
- Process consistency depends heavily on technician skill and alignment accuracy
Impact
- Time-consuming PM process
- Repetitive work for line support technicians
- Possibility of variation between operators
- Difficult to scale if PM demand increases
- Manual documentation adds extra workload

---

## Slide 16

# Project Objective
CURRENT STATE
The PM sensor connection is manual, labour-intensive and prone to human error.
PROPOSED SOLUTION
Integrate a Collaborative Robot (Cobot) arm to automatically handle the sensor data cable plug connection to the OHT.
GOAL
Reduce operational downtime, eliminate manual alignment errors and enable a fully automated, vision-assisted PM workflow.

---

## Slide 17

# Cobot & Vision Candidates
Omron TM series
UR7e
ABB GoFa
ABB PoWa

SICK Smart Vision Guidance
PLOC2D-631C robot guidance with V3D structured-light sensor (~SGD 5,029)
Detects plug position so the cobot aligns and connects automatically
Video demo: PLOC2D-631C | SICK

---

## Slide 18

# Vendor Comparison
| Criteria | OMRON (TM5S) | Universal Robots (UR7e) | ABB (GoFa) | ABB (PoWa) |
| --- | --- | --- | --- | --- |
| Price Range | SGD 100k - 150k | ~SGD 29k - 36k | ~SGD 30k (GoFa13) ~SGD 20k (GoFa5) | ~SGD 20K (PoWa13) ~SGD 15K (PoWa7) |
| Reach | 900 mm | 850 mm | Up to 1300mm Up to 950mm | Up to 1300mm Up to 900mm |
| Weight (arm) | ~23.6 kg | ~20.7 kg | ~48 kg (GoFa 12) | ~41 kg (PoWa 13) |
| Speed | 1.4 m/s | Up to 4 m/s | Up to 2.2 m/s | Up to 5.8m/s |
| Safety | Advanced collision | 17 configurable functions | Advanced torque sensors – all 6 axis | SafeMove (current monitoring) |
| Vision | Built-in / Integrated | Third Party | Third Party ~SGD 5k (SICK Smart Vision) | Third Party Supports plug-and-play flange integration |
| Additional Add-ons | 2x Landmark (~SGD 20); realign OHT ports. | Retrofit port to magnetic plug; vision-guided. | Retrofit port to magnetic plug; vision-guided. | Retrofit port to magnetic plug; minimal deviation. |

---

## Slide 19

# Magnetic Connector for Retrofit
Rosenberger MultiMag 15 (15-way, from RS) • ~SGD 62–67
CFE CXA-0241 (12-pin magnetic) • quote on request
CLE CFA-0241 (12-pin magnetic) • quote on request
Magnetic pogo-pin (8–12P) • ~SGD 3.69/unit

---

## Slide 20

# Retrofit Cost Analysis (304 Vehicles)
| Component / Requirement | Est. Unit Cost | Total Cost |
| --- | --- | --- |
| Magnetic 15-Pin Plug (Premium Rosenberger) from RS | ~SGD 70 | ~SGD 21,280 |
| Magnetic Pogo-Pin Plug (Budget Alternative) | ~SGD 4 | ~SGD 1,216 |
| Custom PCB Bridge Board (per OHT) | ~SGD 3 | ~SGD 912 |
| GRAND TOTAL (Premium Plug) | - | ~SGD 22,192 |
| GRAND TOTAL (Budget Plug) | - | ~SGD 2,128 |

Plus one-time retrofit labor (304 OHTs)
1 hr per OHT × 304 = 304 hrs  •  AE rate ~$18.03 USD/hr ($45k/yr, 4 days/wk @ 12 hrs)  •  Est. labor cost ~$5,481 USD (~SGD 7,344)
Estimated rollout timeline
~51–76 working days (~2.5–4 months) at a pace of 4–6 OHTs/day  •  paced by daily release, not labor capacity (~4–6 hrs/day)

---

## Slide 21

# Magnetic Sourcing & PCB Implementation
| Manufacturer / Brand | Connector Quality | Est. Unit Price |
| --- | --- | --- |
| MultiMag 15, Straight Panel 15 way Magnetic Connector, Brass Contact, PCB | Rosenberger from RS | ~SGD 70.00 |
| [CXA-0241] 12 pin magnetic charging cable connector | CFE Electronic Co.,Ltd | ~To be enquired |
| [CFA-0241] 12 Pin magnetic charging cable connector | CLE TECH CO.,LTD | ~To be enquired |
| 12V 3A Magnetic Pogo Pin Connector 8P-12P Male Female Socket | Standard Pogo-Pin | ~SGD 4.00 |
The Bridge Board: We will design a tiny, custom PCB (~SGD 3 each) to seamlessly map the magnetic pins to our standard wires.

---

## Slide 22

# Connection: Magnetic Plug & Relay Station
ON THE OHT — RELAY SWITCH — PCB
Relay · VHL — VHL Sensor · 10-pin port
Min. 12-PIN MAGNETIC PLUG on the OHT
MAGNETIC PLUG at the workstation
Laptop/Desktop Workstation
Relay · OBS Right — 2-PIN JUMPER — OBS Right · 10-pin port
Relay · OBS Left — OBS Left · 10-pin port
Relay · OBS Center — OBS Center · 10-pin port
Switch selects one sensor at a time
5 of 10 pins used each
Inside each 10-pin port — only 5 pins used
How it works
A single 12-pin magnetic plug on the OHT carries all four sensors' signals (3 each). At the workstation, a 2-pin jumper feeds a relay-switch bank that connects one sensor at a time.
This reproduces the manual plug-in / plug-out routine from one fixed station, no repeated unplugging between ports.
3 signal pins — a unique colour per sensor
2 jumper pins (blue)
5 pins unused

---

## Slide 23

# The Verdict
OMRON (TM5S) — NOT SELECTED
Pro: Excellent out-of-the-box vision integration
Con: Severely breaks the budget at >SGD 100k
Con: Only saves 10 min per PM — weak cost-to-benefit
ABB Series ★ RECOMMENDED
Pro: Highly cost-effective ~SGD 20K
Pro: Flexible 6-axis, configurable functions
Con: Needs 3rd-party vision plugin (e.g., SICK)
RECOMMENDED PATH
The current 4-port plug is too difficult for basic vision systems. Purchase a cost-effective ABB / UR cobot and retrofit the fleet with magnetic plugs (alignment by feel), accounting for the one-time ~SGD 7,344 manpower retrofit cost.

---

## Slide 24

# Return on Investment
~3.0 yr — Payback / breakeven period — SGD 30k investment ÷ SGD 10.1k saved/yr
~SGD 10.1k — Manpower cost saved per year — 416 technician hrs/yr at SGD 24.34/hr
416 hrs — Technician time saved yearly — 30 min × 4 OHTs/day × 208 days
Value beyond direct labor
- Prevents costly port damage from forced manual connections
- Eliminates human verification errors on every PM
- Frees strict technician time for higher-value maintenance
Investment vs alternative
~SGD 30k our recommended path vs SGD 150k for OMRON — ~1/5th the cost, same outcome
Basis: total investment ~SGD 30k (magnetic plug + install manpower + misc) ÷ annual manpower saved ~SGD 10.1k. Savings = 30 min × 4 OHTs/day × 208 workdays at SGD 24.34/hr (line-support salary USD 45k/yr × 1.35, 4 days/wk @ 12 hrs).

---

## Slide 25

(Image-only slide — no text content)